Managalas market access — method, sources and what to be careful of

Managalas Conservation Area · method, sources and cautions · last updated 4 October 2026

Contents

Coordinate reference systems

Digital elevation model

Deliverable 1 — the road network

OpenStreetMap could not be used

What was used instead

Field knowledge overrides the measurement

The supplied alignments, and what upgrading them would buy

Independent check on the network

Deliverable 2 — road distance and service areas

Deliverable 3 — the figures

What the numbers say about the report's claims

Eleven villages, 1,134 households, are not road-served

Data problems found, and what was done about them

Market stop coordinates

Umuwate is in the wrong place, or the wrong zone

Village name spellings differ from the brief

Positions with a wide household spread

Villages in the Ward Profiles with no coordinates

Additional figure — where the road is slow

Cross-reference against the team's planned track list

Why the test is geometric and not by name

Where it stands

Names used twice, and how the pairing was chosen

Four legs where both ends are recorded but nothing joins them

Online data: what is reachable, and what it gave

Spellings and identities confirmed in the field, 3 Oct 2026

The KML sent out for the places we cannot find

The team's placements, read back in

Ogan Village has burned to the ground

Corrections the team made to our own reference data

Road 1, Road 2 and Road 3 are roads, not places

Files

Replaces Figure 8, which plotted village GPS points and joined market stops with straight lines. Every distance here is measured along a route that somebody has actually travelled and recorded.

## Coordinate reference systems

| Purpose | CRS |
| --- | --- |
| All distance, routing and area computation | EPSG:32755 — WGS 84 / UTM zone 55S, metres |
| All delivered GeoJSON and CSV coordinates | EPSG:4326 — WGS 84 lat/lon |

## Digital elevation model

Copernicus DEM GLO-30, 30 m, tiles S09_00_E148_00 and S10_00_E148_00, downloaded from the public AWS mirror. Used for three things: walking time, road gradient, and the hillshade behind both figures.

Walking time is Tobler's hiking function, not a flat speed: W = 6·exp(−3.5·|S + 0.05|) km/h, S being the slope. That is 5.0 km/h on the flat, 2.3 km/h up a 20 % climb and 3.9 km/h down one. A flat 4 km/h would have understated every climb on this plateau.

## Deliverable 1 — the road network

### OpenStreetMap could not be used

Every OSM endpoint is unreachable from the environment this was built in. Tested and refused: overpass-api.de, overpass.kumi.systems, download.geofabrik.de, api.openstreetmap.org, tile.openstreetmap.org. Digitising from satellite imagery was equally impossible — no basemap could be fetched either.

This is stated rather than worked around. The network below has not been cross-checked against OSM, and should be before publication if anyone has unrestricted network access.

### What was used instead

The project's own GPS recordings, which the brief lists as an acceptable source and which are in fact better here: they are the road as actually travelled.

Track names could not be used to identify roads. Locally a foot track is called a road. Seven of the fifteen recordings whose name contains "road" are walked, including Bioi village to Omajeap main road and Kuheine to Dakanatam main road. Classification is therefore by measurement, in this order:

1.  GPS speed, from per-point timestamps. Walked recordings measure 2.7–4.4 km/h median and never exceed 12 km/h; driven ones 11.7–15.1 km/h with half their moving time above 12 km/h. Nothing falls between 4.4 and 11.7, so the cut sits in the middle of an empty band.

2.  Overlap with a proven road — a second recording of a road is a road. This is what identifies Popondetta to Anatua Road: 151 km, no timestamps, running along a GPX driven at 15 km/h.

3.  Gradient, one-sidedly. Every speed-confirmed road is ≤ 15.8 % at the 85th percentile; foot tracks reach 36.9 %. They overlap between 11 and 16 %, so gradient can rule a road out but never in.

4.  The 1973 T683 topographic sheets, where the surveyors drew road, vehicle track and foot track as separate classes. Weakest, since a road may have been built since.

Every feature in mca_roads.geojson carries mode_evidence naming the rule that classified it, plus its measured speed and gradient where those exist.

Result: 478 km of motorable road, 402 km of foot track.

Two promotion thresholds were tightened after the first pass, because the distribution of the evidence showed they were too loose. The overlap shares that promoted a track to a road are sharply bimodal: 23 sit at 98–100 %, which is a second recording of the same road, and 4 sit at 68–85 %, which is a foot track running alongside a road for part of its way. One of the loose four is named Siribu Bush Track, and it was what made Siribu look road-served. The overlap bar is now 0.95. Exactly one feature was promoted by the 1973 rule, at 52 %, and that too was at Siribu; that bar is now 0.70.

Siribu is road-served — but that is established below by someone who has driven the route, on named segments, not by a 68 % overlap share. The tightened bars stand; the evidence for Siribu changed, not the threshold.

surface and tracktype are written as unknown. They are not recorded in the source data and were not going to be guessed.

### Field knowledge overrides the measurement

data/access_overrides.csv records statements from someone who knows the ground, and they outrank the automated rules. data/reference/confirmed_roads.csv does the same thing one level down, for individual recorded segments.

Yoivi has track access only at present, upgradeable to road. Yoivi measures 25 m from a road a vehicle was driven along at road speed, but that is 25 m from a coordinate the brief rounded to two decimal places, about ±1.1 km, so the proximity does not establish access. Rather than move a threshold until the numbers agreed, Yoivi is marked track-only in the data and drawn hollow on both figures. It is removed from the road routing entirely — a stop a vehicle cannot reach cannot serve anyone by road. It remains a market stop: people still walk in, and the walking figures still use it.

Siribu is road-accessible. The route from the Ugunomu junction to Siribu was driven by the project lead in September 2026. The measurement had classified two of its segments as foot track, so they are promoted by name in confirmed_roads.csv rather than by loosening a threshold:

| Segment | Length |
| --- | --- |
| Feeder road to Siribu Village (seg 59) | 1.73 km |
| End of Siribu Village (seg 3) | 1.99 km |

Total promoted: 3.72 km over 2 segments, each carrying mode_evidence = "confirmed by field knowledge: …" in mca_roads.geojson so the promotion travels with the data. With those and the two supplied alignments (2.44 km) the network is 478 km over 32 features.

The Ugunomu→Siribu route measures 3.36 km end to end. This reverses the earlier position here that Siribu had no road access — that conclusion rested on the tightened automatic thresholds alone, and field knowledge outranks them.

No road reaches Kiera or Natanga, confirmed from the ground and recorded in data/reference/footpath_only_villages.csv. The measurement agrees: 2,078 m and 1,702 m respectively to the nearest road.

That does not make them unserved. Every village off the road is reached on foot — the question is how long the walk is, not whether one exists. Kiera and Natanga are a 2.1 km and a 1.7 km walk from the road, which is an ordinary walk to transport anywhere. Both are road-served. road_at_village in the CSV carries the fact that no road runs into the village; access_class carries what that costs in walking.

### The supplied alignments, and what upgrading them would buy

Two further alignments were supplied directly and are held in data/reference/supplied_roads.geojson, separate from anything the speed test established, so their provenance travels with them:

|  | Length | Runs from | To |
| --- | --- | --- | --- |
| Siribu road | 1.24 km | the Ugunomu junction, which is on the main road | south toward Siribu |
| Yoivi road | 1.20 km | the junction near Verayame | south-east toward the village |

The Siribu alignment does continue to the village, contrary to a first reading here that it stopped 1.8 km short. It ends 394 m from End of Siribu Village (seg 3), a 1.99 km segment that reaches within 13 m of surveyed Siribu at a 10.8 % gradient — well inside what a road here does. That segment had been classified a foot track on the 1973 fallback alone, and is one of the two now confirmed driveable.

Only Yoivi is left as an upgrade scenario. What upgrading it would buy:

| Households | As now | If Yoivi upgraded | Gain |
| --- | --- | --- | --- |
| within 2 km of a stop by road | 1,380 (36.0 %) | 1,671 (43.5 %) | +291 |
| within 5 km | 2,155 (56.2 %) | 2,274 (59.3 %) | +119 |
| within 10 km | 2,703 (70.4 %) | 2,703 (70.4 %) | 0 |

Reachability at all is unchanged at 2,703 households: the villages concerned can already get to some road-served stop, just a much farther one. What upgrading buys is proximity, not access — about 290 households brought inside 2 km.

--upgraded on build_access.py writes the second scenario to mca_village_access_upgraded.csv and mca_service_areas_upgraded.geojson. It deliberately does not overwrite mca_market_stops.geojson or method.json, which describe the delivered figures rather than the what-if.

### Independent check on the network

The brief states which villages the road must and must not reach. Nothing about zones was encoded in the method, and the network reproduces the split exactly:

- Must reach — Itokama, Umbuara, Siribu, Koruwo, Tabuane, Kaura, Afore, Manusi: all within 1.4 km of the network, most within 80 m. Siribu itself is 13 m from it once the driven Ugunomu route is counted, which is what the brief says and what the field knowledge confirms.

- Must not reach — Suari, Jaure, Aiari, Gewoya, Toma, Biriri, Kero, Kinando, Borohojo: all 7.9 to 16.8 km away.

Two villages the brief does not mention either way, Kiera and Natanga, are excluded on field knowledge rather than on this test — see above.

No road is drawn into Zones 4, 9 or 10.

## Deliverable 2 — road distance and service areas

road_km is distance along the road, node to node on a graph built by noding the road lines at their intersections. The walk from a village to wherever it meets the road is reported separately as offroad_to_road_m rather than folded in — folding it in made Kaura, itself a market stop 1.4 km off the road, come out as 1.4 km from itself.

A village further than 2 km from the network is reported as road_reachable = no with road_km blank. It is not given a straight-line figure, which would read as a road distance.

Access is graded on the walk to the road, not switched on and off at one line. An earlier version of this method classed a village as unserved the moment its walk passed 2 km, which put Kiera on the wrong side by 78 m and made a 1.7 km walk at Natanga equivalent to the 16.8 km at Aiari. Three classes are used instead, in access_class:

| Class | Walk to the road | Villages | Households |
| --- | --- | --- | --- |
| road-served | ≤ 2.5 km | 35 | 2,703 (70.4 %) |
| long walk to road | 2.5–10 km | 4 | 336 (8.8 %) |
| road not practically reachable | > 10 km | 7 | 798 (20.8 %) |

The 2.5 km cut sits in the widest gap the data offers — Kiera at 2.08 km, then Dareki at 3.19 km — so no village's class turns on a few metres. The 10 km cut is the other end of the same judgement: beyond it the walk stops being a commute. Both are stated here rather than tuned, and offroad_to_road_m is in every row so a different cut can be applied without rerunning anything.

The road and walking graphs are built by noding the recordings and then bridging endpoint gaps under 450 m, because separate recordings of the same road stop and restart a few metres apart. Without it the Siribu spur was an island in the graph and no route to it existed at all. Bridging is reported every run: 2 gaps, 0.44 km on the road graph; 46 gaps, 9.52 km on the walkable one. Anything wider than 450 m is left unjoined rather than invented.

mca_service_areas.geojson holds 15 polygons — three bands × the five stops a vehicle can reach. Yoivi has none, because it has no road access. Each is a corridor 1 km wide centred on the road, not an area claim. The width is a drawing decision so the bands can be read on a 55 km-wide map; at the 300 m first used they were invisible under the road line. Do not read them as "everywhere within 500 m of the road is served".

## Deliverable 3 — the figures

fig_mca_map.png and fig_market_reach.png, both 9×7 inches at 200 dpi, DejaVu Sans 9 pt, no title or figure number inside the image. Palette as specified: #1F4E79 circuit and stops, #C4C4C4 other villages, #B03A2E villages with no road access.

Zone boundaries are not drawn. No polygon for them exists in the supplied data and the brief says to leave them out rather than approximate them.

The villages drawn red on Figure A are those more than a 2.5 km walk from a road, computed, not a fixed list of zones. Dareki and Sigara are red and are not in Zones 4, 9 or 10; Kiera and Natanga are Zone 3 with no road at the village and are not red, because the walk is short.

A third figure, fig_road_speed.png, was added on request — see below.

## What the numbers say about the report's claims

| Claim in the report | What the road network gives |
| --- | --- |
| 3,059 households, 87.7 %, served by the circuit | 2,703 of 3,837 households, 70.4 %, are road-served |
| — | within 2 km by road: 1,380 hh (36.0 %) |
| — | within 5 km: 2,155 hh (56.2 %) |
| — | within 10 km: 2,703 hh (70.4 %) |
| Siribu covers Zones 3 and 5; every village within 4.0 km | Coverage yes, distance no. Siribu is the nearest stop for 7 villages and 399 households, spanning both zones. But only Ugunomu (3.36 km) is inside 4.0 km: Natanga is 5.30, Dea 5.63, Jorua 6.85, Kiera 7.00 and Koruwo 9.58. Kiera and Natanga add a 2.1 km and 1.7 km walk on top, since no road runs into either village. |

Road distance runs a median 1.11× the straight line, so straight-line figures are optimistic throughout — as the brief anticipated. The ratio fell from 1.29× when the Siribu spur entered the network: the Zone 3 and 5 villages no longer have to route the long way round to Kaura or Umbuara.

### Eleven villages, 1,134 households, are not road-served

A long walk to the road — 2.5 to 10 km, 4 villages, 336 households

Dareki (145, 3.2 km), Biriri (82, 8.2 km), Toma (73, 7.8 km), Sigara (36, 6.9 km). These can reach the road on foot, but not as part of a market day.

Beyond 10 km, 7 villages, 798 households

Borohojo (271, 10.4 km), Gewoya (188, 10.5 km), Aiari (109, 16.8 km), Kero (89, 11.0 km), Jaure (59, 14.9 km), Kinando (49, 11.1 km), Suari (33, 14.9 km).

Two are worth separate attention because the brief asks:

- Toma — 7.8 km to the nearest road; 2.8 h walk to Afore.

- Biriri — 8.2 km to the nearest road; 4.4 h walk to Yoivi, which is itself a track-access stop, so this is a walk to a market a vehicle cannot reach either.

Walks are routed over the recorded foot-path network, which the gap bridging joined up; only Aiari, Jaure and Suari (Zone 4) fall back to a straight line, and those three are optimistic because a real route will be longer. walk_basis in the CSV says which basis each village used.

## Data problems found, and what was done about them

### Market stop coordinates

Four stops are quoted to four decimal places; Yoivi and Siribu to two, which is about ±1.1 km. It showed: the brief's Siribu sits 2.0 km off the road while the surveyed Siribu is 13 m from it.

- Siribu — the surveyed position is used instead, 2,072 m from the one given.

- Yoivi — not in the survey file at all. The brief's rounded position is used as given. The nearest surveyed village is Verayame, 2.2 km away. Yoivi should be surveyed before these figures are published.

### Umuwate is in the wrong place, or the wrong zone

Listed in Zone 1 (the Itokama cluster) but its coordinates, −9.135433 / 148.401440, put it beside Afore in Zone 7b, roughly 15 km from every other Zone 1 village. One of the two fields is wrong. It is carried through as supplied and flagged here rather than silently moved.

### Village name spellings differ from the brief

The brief says Yaure, Kinado, Boroheje; the file says Jaure, Kinando, Borohojo. The file's spellings are used. Aiari, Gewoya, Toma, Biriri, Kero and Suari agree.

### Positions with a wide household spread

Ten villages have household points scattered over more than 10 km. The median is robust so the centroids are usable, but these are not surveyed village centres and should not be presented as such. position_spread_km is carried in the CSV for every village.

| Village | Zone | Spread |
| --- | --- | --- |
| Suari | 4 | 37.9 km |
| Jaure | 4 | 33.1 km |
| Niniuri | 7b | 22.1 km |
| Dea | 5 | 20.1 km |
| Kwae | 1 | 16.5 km |
| Gewoya | 9 | 15.9 km |
| Natanga | 3 | 14.4 km |
| Biriri | 9 | 14.1 km |
| Kawowoki | 7a | 13.6 km |
| Tabuane | 5 | 11.0 km |

Suari and Jaure are the worst and both are in Zone 4 — which has no road, so their access figures rest on a position that could be tens of kilometres out.

### Villages in the Ward Profiles with no coordinates

The Ward Profiles of August 2026 record 73 villages and 3,489 households in the Conservation Area. The survey file holds 46 villages, so 27 villages have no coordinates and could not be placed.

They could not be located: no gazetteer or OSM lookup was possible (see above), and inventing positions was not an option. They are absent from both figures and from the access table.

One inconsistency to resolve at source: the survey file's own est_hh sums to 3,837, which is 348 more than the Ward Profiles' 3,489 for the whole Conservation Area — even though the file covers only 46 of 73 villages. The two household figures are not reconcilable as they stand, so the percentages above are computed against the 3,837 in the file and should be revised once that is settled.

## Additional figure — where the road is slow

fig_road_speed.png and mca_road_speed.geojson, added on request.

Driven recordings are cut into point-to-point steps and attached to 500 m chunks of road. Stopped time (< 3 km/h) is excluded, as are recorder dropouts. A recording can be part driven and part walked — one is called track and road to Afore and is exactly that — so a stretch counts as driven only if the vehicle reached road speed within a two-minute window; otherwise a walked leg reads as 3 km/h road.

Coverage is the dominant limitation: only 33 km, over 75 chunks, has enough passes to measure, from 5 driven recordings in 3 files. Chunks below 8 observations are drawn grey and left unclassified. Most of the network has never been driven with a GPS running.

This pass runs on the 472 km network that the speed and overlap tests classified, not the 478 km delivered in mca_roads.geojson: the 3.72 km confirmed by field knowledge and the 2.44 km supplied as alignments carry no GPS timing, so there is nothing on them to measure either way.

Over the measured 33 km: median 13.2 km/h, slowest chunk 6.2, fastest 23.7.

Terrain does not explain where it is slow. Spearman correlations against median speed:

|  | rho |
| --- | --- |
| mean gradient | −0.00 |
| steepest gradient | +0.04 |
| sinuosity | −0.20 |
| elevation | −0.32 |

Gradient is effectively unrelated to speed. The only appreciable relationship is with elevation, and that is confounded rather than causal: the fast chunks are the coastal approach near Popondetta (median 89 m) and the slow ones are up on the plateau (median 776 m). Those are different roads, not the same road behaving differently with altitude.

The slowest fifth does sit on steeper ground than the fastest fifth (17.6 % vs 11.8 % steepest gradient) and its mean gradient is somewhat higher (8.2 % vs 5.9 %) — but across all 75 chunks the correlation is nil, so this is a difference between two small groups rather than a gradient effect. Taken together this points at surface condition rather than topography — which this data cannot measure. Confirming it needs either a surface survey or repeated passes over the same chunks in different seasons.

## Cross-reference against the team's planned track list

NAMES_OF_FOOT_TRACKS_IN_MANAGALAS_CONSERVATION_AREA.docx (3 Oct 2026) names 80 routes the team identified to walk: 62 foot tracks, 16 feeder roads and 2 national highways. Transcribed verbatim into data/reference/planned_tracks.csv, broken into 110 two-place legs, and tested against the 124 recordings in data/processed/tracks.gpkg by scripts/cross_reference_tracks.py.

### Why the test is geometric and not by name

Matching names would have been quick and wrong. Thirty-eight of the 124 recordings carry no route name at all — they are russell muraba track record july 10 july (seg NN) — and the ones that do carry names use spellings the list does not (Avaru/Awaru, Kueno/Koeno, Jorura/Jorua, Kiara/Kiera, Turatana/Turatan, Niniri/Niniuri). So a leg passes only when

1.  both of its ends sit within 500 m of a recorded line, and

2.  the noded network routes between them at no more than 1.8 × the straight line (2.5 × under 2.5 km, where terrain detours are proportionally larger).

Test 2 is what stops a leg being called done because its two ends happen to lie on the network at opposite ends of the plateau, which is the error a proximity-only test makes. Where a recording is named for a leg, the name is reported as corroboration beside the measurement, never in place of it.

### Where it stands

|  | legs | routes |
| --- | --- | --- |
| recorded | 42 | 28 |
| partly recorded | 20 | 16 |
| not recorded | 6 | 5 |
| place not located | 42 | 31 |

The largest single blocker is not walking, it is the gazetteer. Thirty-five place names in the list appear in nothing we hold — neither MCA_Village_Locations.csv nor the GPS recordings — so 31 of the 80 routes cannot be assessed at all. They are listed at the foot of TRACK_CHECKLIST.md for the team to position.

### Names used twice, and how the pairing was chosen

Several names sit on two places kilometres apart: two Kauras 12 km apart, two Kuharas, two Sigaras 10.9 km apart, two Tomas 5.1 km apart, two Siribus. Of the candidate positions for a leg's two ends, the nearer pairing is taken — a list of village-to-village walks is naming adjacent places, so the close pairing is the one it is about — with the surveyed gazetteer preferred over the inferred position on a tie. Every leg decided this way carries the flag and the distance in the checklist, so the assumption can be rejected.

Two positions were fixed from direct evidence rather than left unlocated, both recorded in data/reference/place_aliases.csv:

- Umuate — the gazetteer's Umuwate at 148.4014 is 19 km from the endpoint of Davids_track_record_from_Itokama_to_Umuate and cannot be this place. The recording's endpoint, −9.2637 / 148.2370, is used. This is the same Umuwate position already flagged above as internally inconsistent.

- Umbara — distinct from Umbuara, and the list uses both. Umbuara is in Zone 2 beside Anatua and Gadzot; Umbara/Ombara is in the east beyond Kuheine, fixed at 148.4675 by the endpoint of Trail to Umbara village (seg 11).

### Four legs where both ends are recorded but nothing joins them

Tahama–Toma, Toma–Itokama, Afore–Toma and Bareji–Toma. All four are into Toma: the recordings reach the Toma area but the network is in two pieces there, so Toma has recordings around it and no recorded connection through it. That is a real gap and not a bridging artefact — 450 m gap bridging was already applied before the test.

The other two not-recorded legs are the far south-west, Aiari–Suari and Suari–Jaure, where there is nothing recorded within 10 km of either end. Zone 4 is unvisited: Aiari (109 households), Jaure (59) and Suari (33) have no GPS coverage at all.

### Online data: what is reachable, and what it gave

Asked on 3 Oct 2026 whether public sources had been used, the answer was no and the reason was the network policy, which was retested rather than assumed: Nominatim, Overpass and GeoNames all answered 403 at the gateway. The policy was opened on 4 Oct and retested again. Where it stands now:

| source | result |
| --- | --- |
| nominatim.openstreetmap.org | open — village-level coverage of Oro, and good |
| www.geonames.org (search page) | open — thin for this area, useful as a second opinion |
| api.geonames.org | 403 — the HTML search page is used instead |
| overpass-api.de | still denied |
| download.geofabrik.de | still denied |

So no bulk OSM extract and no Overpass queries; the road network remains entirely the project's own recordings. But name lookups now work, and scripts/geocode_unlocated.py ran every unlocated place through both sources.

Nothing was accepted on a name match. GeoNames' only "Itokama" in Papua New Guinea is an airport at 147.30 in Central Province, 106 km from the Itokama this project is about — accepting hits on name alone would have moved a village across the country. So every candidate is reported in geocode_candidates.csv with its administrative division, its feature class and its distance from the anchor the route's own neighbours imply, and only hits the route geometry corroborates were taken. The filter rejected an Asafa 406 km away in Eastern Highlands, a Mount Embi 656 km away in Enga, a Kokoro Airport 238 km away in Central Province and a Damera 551 km away in Western Province.

Twelve places were positioned this way, taking the unlocated count from 35 to 18 and the recorded legs from 44 to 51. The strongest are the ones a recording already names:

- Aguido = Aguido Primary School, 200 m from where the route implied, and named in Vereyame_connects_Yoivi,_Aguido_Primary_School_and_Kuheine — so R02 turns out to be already walked.

- Bodoimo, 700 m from where the route implied, and named in Korouwo_to_Anatua_via_Bodoimo — F55 likewise.

- Kokoro, an OSM village 3 km south of Anatua, which settles the 3 Oct query: it is not Karokora beside Afore.

- Babarobo sits almost exactly on the Biriri–Safia line, which is the leg the list puts it in.

- Embi and Warisota come out 1.7 km apart and are consecutive in "Numba-Umasi-Embi-Warisota", which is better evidence than either hit alone. Embi's longitude was the thing missing on 3 Oct.

- Gora > Asafa > Sairope is a consistent westward chain.

Two carry a caveat in place_aliases.csv: Warisota is ambiguous — GeoNames has another 13 km north and OSM a New Warisota 45 km west — and Peroroda is 20 km from its anchor, though that anchor is the weak Bareji cluster so the distance proves little either way.

Thirteen names are in neither gazetteer and still need the team: Aputuara, Emoriva, Gorobuna, Haraja, Korokoro, Singata, Tedebedi, Umasi, Vovosik, Wayei, Water Crossing, and the Organ and Old Cardamom factory ends of Road 1 and Road 2.

### Spellings and identities confirmed in the field, 3 Oct 2026

Avaru = Awaru. Kueno = Koeno, also written Kweno. Kiara = Kiera, and not Kira. Jorura is probably Jorua, treated as the same place. Kuai = the gazetteer's Kwae, which resolves three routes. Umbara is not Umbuara, confirming the reading taken above. Tetebedi/Tedebedi is outside the conservation area, so "Anatua-Tetebedi" and "Kuai-Tedebedi-Emoriva" run beyond the boundary.

### The KML sent out for the places we cannot find

scripts/make_unlocated_kml.py writes unlocated_places.kml: a pin per unlocated place to be dragged onto the real village in Google Earth, over two locked reference folders — the villages we do hold, and the recorded tracks.

No pin is a position we hold, and each says so. Each starts at the midpoint of the places the list itself names beside it, which is a far better opening guess than the middle of the map: "Umbuara-Singata" and "Singata-Gora" between them put Singata near the Umbuara–Gora line. Where every neighbour is also unknown the anchoring runs again over the pins it has already guessed, so "Numba-Umasi-Embi-Warisota" walks outwards from Numba instead of collapsing to the centre; how many steps from solid ground each pin is is stated on it. Four pins — Road 1, Road 2 and the Organ and Old Cardamom factory ends of them — have no located place anywhere in their route and say that the position is meaningless.

### The team's placements, read back in

scripts/ingest_placed_kml.py reads a returned KML or KMZ, works out which version of the pin file it was edited from — the first batch came back against the 30-pin file while the checklist had already moved to 18 — and writes data/reference/placed_by_team.csv.

That file is read ahead of the gazetteer and ahead of our own inference: a position someone put on the map from knowledge of the ground outranks both, and a name found there returns that one position with nothing for the leg to choose between.

A pin is only taken as placed if it moved from the guess it was sent out at, by more than 50 m. A pin left sitting on its guess says nothing — the guess was ours — and recording it as a field position would launder our own arithmetic into evidence. In the first batch 17 of 30 moved and 13 were left, and the 13 are still treated as unplaced.

The round trip validated the geocoding. Five of the places the team moved were ones the public gazetteers had already given us, and all five agree:

| place | field placement vs OSM |
| --- | --- |
| Emo | 0.05 km |
| Bodoimo | 0.05 km |
| Kokoro | 0.05 km |
| Banderi | 0.45 km |
| Ufia | 0.60 km |

Independent agreement at that distance on five of five is good reason to trust the remaining OSM positions, which are flagged in place_aliases.csv but had no other corroboration. Tedebedi moved 21 km west to 148.070, which is outside the conservation area and so agrees with what the team said about it on 3 Oct.

After this round: 55 of 110 legs recorded, 34 partly, 12 not recorded, 9 waiting on a position — six names, down from 35. Only Aputuara, Damara, Emoriva, Umasi, Vovosik and Water Crossing are still unknown.

One placement to query: Road 1 and Organ came back 52 m apart, so R05 "Road 1 - Organ Village" has no leg to walk. That may be right — the road may simply end at the village — or both pins may have been dropped on the same feature.

### Ogan Village has burned to the ground

Reported on 4 Oct 2026. "Organ" in the track list is a typo of Ogan — which is confirmed beyond doubt, because the position the team placed for it landed 26 m from the gazetteer's Ogan.

This matters to Deliverable 2 and not only to the track list. Ogan is carried through mca_village_access.csv as 69 households, road-served, 1.48 km by road to Afore — 2.6 % of the 2,703 road-served households and 1.8 % of the 3,837 total. The access figures therefore count a village that no longer stands.

They are flagged, not corrected. Whether the households have relocated, and where to, is not known here, and deleting 69 households would be as wrong as leaving them: the people exist either way, and where they now are is what decides their access. Recorded in data/reference/village_status.csv, and the one question that would resolve it is where those households have gone.

### Corrections the team made to our own reference data

The pin file went out with the villages and recordings we hold as locked reference folders, mostly so a pin could be placed relative to its neighbours. It came back with comments on them, and those were the most valuable thing in the file — none was findable from the geometry:

| marked | what it was | what was done |
| --- | --- | --- |
| Suari | "This was WRONG" | The gazetteer puts Suari 73 m from Jaure, which is why F37 produced a leg with no length. The team moved it 10.5 km, to 148.17331 / −9.36637 |
| Ondoro | "This is WRONG. Ondoro is roadside" | The inferred cluster at 148.4677 is not it. Moved 2.4 km, to 148.46913 / −9.11472 |
| Niniuri | moved without comment | Moved 2.9 km, to 148.41433 / −9.12854 — which puts it 300 m from Simeon's Place, agreeing with "the village is Niniuri" |
| Kawowoki | moved without comment | Moved 130 m |
| Gadzot | moved as well as commented | Moved 800 m, landing 70 m from the gazetteer's Gajot — which is the merge confirmed by position, not just by spelling |
| Vouka | "But NO VILLAGE HERE" | Both inferred Vouka clusters rejected. Dropped |
| Gadzot | "mispelling of Gajot" | The inferred Gadzot and the gazetteer's Gajot are one village 0.8 km apart |
| Semehara | "typo of semahara" | The gazetteer spelling is the typo. Outputs should read Semahara |
| Sigara | "the path ends at the main village, but the point is in a hamlet of Sigara. Near, but across a river" | Confirmed: Track from Kaura to Sigara Village ends 1,281 m west of the gazetteer point. "Sigara" now means the main village |

The rejected originals are listed in data/reference/bad_positions.csv and filtered out by load_places(), so nothing downstream can use them again; the corrected positions sit in placed_by_team.csv, which is read first.

Five of those corrections were nearly lost. The reference folders went out labelled "do not move", and the first pass of ingest_placed_kml.py diffed positions only in the editable folder. The team corrected five villages in the reference folder regardless — which is exactly what should happen, and the most valuable thing the file can come back with. The ingest now diffs every folder and reports corrections to our own data separately, and the folder is relabelled "correct any that are wrong".

None of the four position corrections changes an access class, which was checked rather than assumed:

| village | households | off-road, was → now | class |
| --- | --- | --- | --- |
| Suari | 33 | 14.93 → 20.32 km | unchanged, beyond 10 km |
| Niniuri | 20 | 0.35 → 0.14 km | unchanged, road-served |
| Kawowoki | 277 | 0.34 → 0.47 km | unchanged, road-served |
| Gajot | 89 | 0.17 → 0.19 km | unchanged, road-served |

So the headline access figures survive the corrections. The one field fact that does reach them is Ogan burning down.

The Sigara correction had a consequence worth recording. Once "Sigara" meant the main village, "Kaura-Sigara" flipped from recorded to partly recorded — because the pairing rule chose the northern Kaura over the surveyed one on a 0.3 km margin. That rule was wrong: it let distance outrank provenance, when the team had just finished showing that the GPS inference contains errors the gazetteer does not. Source now outranks distance — the team's placements, then the surveyed gazetteer, then our inference — with distance only breaking ties inside a tier. Kaura-Sigara is recorded again, routing in 9.0 km against 7.0 km straight, which matches the recording that is named for it.

### Road 1, Road 2 and Road 3 are roads, not places

"Road 3 is the road to get there"; "Road 1 ... passes through Ogan village". The leg model was wrong for these three, which were being asked to join two points when one of the points was the road itself. Their entries now carry the single place each road serves, and the test is whether a recorded motorable road reaches it. Road 3 reaches Dareki (162 m) and Road 1 reaches Ogan (13 m); Road 2 has no recorded road within 1.0 km of the old cardamom factory.

Three other entries turned out to have no leg to walk: Aiari-Water Crossing ("This is Aiari village. I dont know why water crossing is a place name"), Simion's place which is Simeon's house rather than a village, and Ufia which "is used somewhat interchangeably" with the gazetteer's Madokoro — the team's pin landed 3 m from it.

Emoriva is "a contraction of Emo River. Not here and not inside MCA", which fits: Tedebedi was placed 2 km from the Emo River airstrip at 148.04, so F29 runs off the plateau to the west. And there are two Emos: the one the list means in "Avaru-Sigara-Emo" is the eastern village at 148.525, since the Emo River one is 50 km west of Sigara and could not be on that route.

## Files

| File | What it is |
| --- | --- |
| mca_roads.geojson | 478 km motorable road, with per-segment classification evidence |
| mca_market_stops.geojson | the six stops, with road or track access marked |
| mca_village_access.csv | 46 villages: nearest stop, road km, straight km, walking hours, access_class, road_at_village, and the caveats per row |
| mca_service_areas.geojson | 15 corridor polygons, 2/5/10 km road distance × the 5 road-served stops |
| fig_mca_map.png | Figure A — the map |
| fig_market_reach.png | Figure B — market reach |
| fig_road_speed.png | Additional — measured road speed and its relation to terrain |
| mca_road_speed.geojson | 500 m chunks with speed, gradient, sinuosity and observation counts |
| method.json | CRS, DEM, thresholds |
| TRACK_CHECKLIST.md | the team's 80 planned routes, each leg marked recorded / partly / not recorded / place not located |
| track_checklist.csv | the same, one row per leg, with the measurement behind every call |
| unlocated_places.kml | a draggable pin per place we cannot find, over the known villages and the recorded tracks |
| geocode_candidates.csv | every public-gazetteer hit considered for an unlocated place, with the division, feature class and distance that decided it |

Positions the team placed by hand live in data/reference/placed_by_team.csv and are read ahead of everything else. Their comments are kept verbatim in data/reference/field_comments.csv, positions they rejected in data/reference/bad_positions.csv, and villages whose status has changed in data/reference/village_status.csv.

Scripts that produced them, in order: classify_mode.py → build_access.py → make_access_figures.py → road_speed.py

cross_reference_tracks.py produces the checklist and is independent of the access pipeline; it reuses that pipeline's graph noding and gap bridging. make_unlocated_kml.py writes the pin file and geocode_unlocated.py the gazetteer candidates; both read the checklist, so run it first.

Field-knowledge overrides live in data/access_overrides.csv (stop access), data/reference/confirmed_roads.csv (individual segments confirmed driveable) and data/reference/footpath_only_villages.csv (villages with no road running into them). The planned list and its spelling reconciliation live in data/reference/planned_tracks.csv and data/reference/place_aliases.csv.